SENTINEL AI

Hosting & Deployment Proposal

10,000-Camera Campus Surveillance Deployment

Prepared for: Bid / Project Manager

Date: 12 July 2026

Currency: Indian Rupees (₹)  ·  Assumed FX: US$1 = ₹85

Classification: Commercial-in-Confidence

# 1.  Executive Summary

This proposal sets out the hosting and deployment options for running Sentinel AI across a fleet of 10,000 cameras, with indicative costs in Indian Rupees, and the advantages and disadvantages of each option.

At this scale the deciding constraint is network bandwidth: 10,000 high-definition camera streams generate roughly 30 Gbps of continuous video, which cannot be economically shipped to a central cloud. The recommended architecture is therefore Edge inference at each site plus a central Cloud control plane (Option A) — video is analysed where it is captured, and only events, metadata and short clips travel over the network.

Four options are compared. Costs are planning-grade estimates (typically accurate to within ±40%) and move with frame rate, resolution, video-retention period, GPU throughput actually achieved, and the FX rate on the day.

# 2.  Scope & Key Assumptions

| Parameter | Assumption used |
| --- | --- |
| Camera count | 10,000 cameras |
| Stream profile | ~1080p H.264, analysed at ~5–10 fps (surveillance rate, not 30 fps) |
| Analytics stack | Detection + tracking + person Re-ID (OSNet) + fall / running / violence / loitering |
| GPU throughput | ~15–30 cameras per modern GPU (T4 / L4 / A10-class) once optimised (TensorRT / DeepStream) |
| GPUs required | ~500 GPUs of compute for the full fleet |
| Total video bandwidth | ~30 Gbps continuous if centralised |
| FX rate | US$1 = ₹85 (state actual rate at contract time) |
| Video retention | Full recording kept on-site NVRs; only event clips + metadata held centrally |

Important: the current Sentinel build is a single-node, CPU-only prototype that processes one video stream at a time. Reaching 10,000 cameras requires a productisation build-out (multi-GPU optimised pipeline, a database-backed camera registry, and a networked licensing service). That one-time engineering effort is itemised in Section 5.

# 3.  Recommended Architecture

Edge + Cloud (hybrid). GPU edge servers are installed at each campus/site and run all video analytics locally. A lightweight central cloud hosts the dashboard, licensing/activation, cross-site search, the metadata database, clip storage and reporting. Raw video never leaves the site in bulk; the wide-area network carries only a small stream of events and on-demand clips.

This is the same pattern used by large commercial platforms (e.g. Verkada, Avigilon, and NVIDIA DeepStream deployments) precisely because it removes the 30 Gbps bandwidth problem and lets the system scale one site at a time.

# 4.  Hosting Options — Cost, Advantages & Disadvantages

All figures in ₹, planning-grade. "Crore" = ₹1,00,00,000; "Lakh" = ₹1,00,000.

Option A — Edge Inference + Cloud Control Plane    ★ RECOMMENDED

Analytics run on GPU servers at each site; a lean cloud hosts UI, licensing, metadata and clips.

| One-time (capex) | ₹21–43 crore (GPU edge servers across ~100 sites; ₹12.75–25.5 lakh per site box) |
| --- | --- |
| Recurring (monthly) | ₹40–90 lakh (site power & cooling + cloud control plane ₹1.7–12.75 lakh + storage + operations) |
| Indicative 5-year TCO | ≈ ₹55–95 crore |

Advantages

- ＋  Eliminates the 30 Gbps bandwidth problem — only events/clips leave each site.

- ＋  Scales linearly: add sites independently without re-architecting.

- ＋  Lowest long-run cost; hardware is owned, not rented.

- ＋  Resilient — a site keeps working even if its internet link drops.

- ＋  Data-residency friendly (video stays on-premise) — good for schools/privacy.

Disadvantages

- －  High upfront capital cost and hardware procurement lead time.

- －  Physical installation and maintenance at many sites (field ops).

- －  You own hardware refresh (typically every 4–5 years).

Option B — Fully Centralised Cloud GPUs    Not advised at this scale

All camera video is streamed to the cloud; ~500 rented cloud GPUs run the analytics centrally.

| One-time (capex) | Minimal (rental model, no hardware purchase) |
| --- | --- |
| Recurring (monthly) | ₹1.5–3.5 crore (≈ ₹1.27–3.15 crore GPU rental + control plane + storage + wide-area bandwidth) |
| Indicative 5-year TCO | ≈ ₹110–200 crore |

Advantages

- ＋  No upfront capital outlay; fastest to stand up.

- ＋  Elastic — scale GPUs up or down on demand.

- ＋  No on-site hardware to maintain.

Disadvantages

- －  Requires ~30 Gbps sustained ingest — often infeasible and very costly.

- －  By far the highest recurring cost; ~2× the 5-year TCO of Option A.

- －  Central point of failure; a link outage blinds an entire site.

- －  All video leaves the premises — weaker privacy/data-residency posture.

Option C — Central On-Premise Data Centre    Viable for few, large sites

You buy and run ~500 GPUs in your own (or colocated) data centre; video is aggregated there.

| One-time (capex) | ₹42–50 crore (GPUs + power/cooling infrastructure + networking) |
| --- | --- |
| Recurring (monthly) | ₹25–70 lakh (power, cooling, colocation, operations staff) |
| Indicative 5-year TCO | ≈ ₹60–95 crore |

Advantages

- ＋  Full control over hardware, data and security.

- ＋  Cheaper than cloud rental over a 5-year horizon.

- ＋  Centralised operations — one location to manage, not a hundred.

Disadvantages

- －  Still must transport ~30 Gbps of video to the data centre — the core bandwidth problem remains unless sites are few and large.

- －  Large upfront capital plus power/cooling build-out (~200–300 kW).

- －  You carry data-centre operations, redundancy and disaster recovery.

Option D — Pure Per-Camera SaaS (no edge)    Not suitable at 10k

Fully hosted service, priced per camera, with no on-site inference hardware.

| One-time (capex) | Minimal |
| --- | --- |
| Recurring (monthly) | Approximates Option B + per-camera onboarding |
| Indicative 5-year TCO | Not recommended for 10k raw-video cameras |

Advantages

- ＋  Simplest commercial model (predictable per-camera fee).

- ＋  Works well for small deployments or cameras that pre-process on-device.

Disadvantages

- －  Inherits Option B's bandwidth and cost problems at 10,000 cameras.

- －  Only realistic with AI-capable cameras or edge pre-processing — extra hardware cost per camera.

# 5.  Side-by-Side Comparison

| Option | Upfront (capex) | Monthly | 5-year TCO |
| --- | --- | --- | --- |
| A · Edge + Cloud ★ | ₹21–43 crore | ₹40–90 lakh | ₹55–95 crore |
| B · Cloud GPUs | Minimal | ₹1.5–3.5 crore | ₹110–200 crore |
| C · On-prem DC | ₹42–50 crore | ₹25–70 lakh | ₹60–95 crore |
| D · Per-camera SaaS | Minimal | ≈ Option B | Not advised |

## One-time productisation / build-out (applies to all options)

The prototype must be hardened into a fleet product: an optimised multi-GPU inference pipeline (TensorRT / DeepStream), a database-backed camera registry replacing flat JSON, and a hardened, server-side licensing/activation service. Indicative effort: a team of 6–10 engineers for 9–12 months.

| Build-out item | Indicative cost |
| --- | --- |
| Engineering team (6–10 engineers, 9–12 months) | ₹1.5–3 crore |
| Licensing hardening (production keypair, online activation, heartbeat) | included in above |
| Pilot / proof-of-concept site (1 site, ~100 cameras) | ₹15–30 lakh |

# 6.  Licensing & Security Readiness

Sentinel's per-client governance was tested live and works as designed: the developer-set camera limit and per-feature restrictions are enforced on the user, and changes take effect immediately without a server restart.

- Camera-count limit — enforced (exceeding the licensed count is rejected).

- Feature restrictions — enforced (features outside the licensed tier return "not included").

- Immediate effect — a re-issued licence is picked up on the next request, no restart.

Before a commercial sale, two hardening items are required and are included in the build-out above:

- －  Replace the current locally-generated developer keypair with a production keypair held offline / in an HSM.

- －  Move licence enforcement to an online activation + periodic heartbeat model, so the protection cannot be trivially removed from the shipped software or bypassed by rolling back the system clock.

# 7.  Recommendation

Adopt Option A (Edge inference + Cloud control plane). It is the only option that resolves the bandwidth constraint cleanly, has the lowest 5-year total cost of ownership, scales site-by-site, and keeps video on-premise for privacy. Budget the GPU fleet as capital expenditure (₹21–43 crore) and steady-state running cost at roughly ₹40–90 lakh per month, plus the one-time productisation build-out (₹1.5–3 crore) and a paid pilot site to de-risk the rollout.

All costs are planning-grade estimates for budgeting only, denominated at US$1 = ₹85, and will vary with frame rate, resolution, retention period, realised GPU throughput, vendor pricing and the prevailing exchange rate. A firm quotation requires a site survey and a confirmed technical specification.